TRANSFORMER EXCITATION CHARACTERISTICS

This simulation is designed to illustrate the current inrush phenomenon in a transformer when it is switched to an ac supply. The transformer is modeled by a series resistance and leakage inductance and by a nonlinear magnetizing inductance; the nonlinearity is expressed by a relationship relating the magnetizing current Im and the flux λ of the form

Im = aλ + bλ9

The flux λ is obtained by integrating the voltage Vm since

Vm = 1/ωο*dλ/dt

Core loss is represented by a shunt resistive branch Rm. The model is described in a per-unit system based on the transformer rating.

Suggested studies:

- 1. Obtain plots of Vin, λ, and Iin versus time for various levels of the applied   voltage (Vs=0.5, 1.0, 1.5).

2. Perform a Fourier decomposition of the input current when Vs=1.0 and phase=0o.

3. Plot the magnetizing curve, i.e. λ versus Im; also plot λ versus Iin (Vs=0 and  phase=0o).

4. Examine and compare the effects of :

a) the time of switching by adjusting the phase angle (90o and 0o)

b) the residual flux by adjusting the IC on the integrator

c) the presence of a dc component by including a dc offset in the sinusoidal input excitation

d) a load represented by an RL-LL series circuit