Three-bus power flow

The figure shows a single-line diagram for an elementary 3-bus power system (N =3). Bus 1 is the slack bus; bus 2 is a voltage-controlled (PV) bus; bus 3 is a load (PQ) bus.

Phasors:

Vk  =  Vk ejdk                                                       m = 1…..N

Bus equations:

∑ Ykm Vm = ∑ (Gkm + jBkm)Vm             m = 1…..N

Bus Powers:

Sk = Pk + jQk = Vk Ik* = ∑ (Gkm –jBkm)(Vk Vm*)]

= ∑ [(Gkm –jBkm)Vk Vm ej(dk-dm)]

= ∑{Vk Vm (Gkm-jBkm) [(cos(dk-dm) + jsin(dk-dm)]}

m = 1…..N

Pk =  Gkk Vk2 + Vk ∑Vm[Gkm cos(dk-dm) +Bkm sin(dk-dm)]

Qk = -Bkk Vk2 + Vk ∑Vm[Gkm sin(dk-dm) -Bkm cos(dk-dm)]

m =1 ...k-1, k+1…N